Wipro Service Delivery

Standard of Procedure

for

Port Reset

Network-Data/20-June-18/Process

Table of contents

1.	Purpose	3

2.	Business as Usual (BAU) Change	3

2.1 Port Reset	3

3.	Roles & Responsibility	3

4.	Environment	3

5.	Definitions & Acronyms	3

6.	References	3

7.	Implementation Steps	3

8.	Amendment History	5

# Purpose

The Purpose of this Steps of Procedure (SOP) is to:

- Identify the steps for implementation of BAU Changes.

# Business as Usual (BAU) Change

- 2.1 Port Reset

This activity is useful to make the interface active/up if it’s down or in err-disabled mode.

The following steps provide a high-level overview that outlines Port Reset request to completion.

Port Reset:

- A service request is to be opened with the YYZ Service Desk for Port Reset

- Request assigned to Service Provisioning to provide the switch interface(s) details for the cable/Data Jack id of the port to be reset by network team,

- if switch and port details already available in the ticket description it will be assigned to Network-Data team.

- Tam will provide Switch and interface details to Reset the port to Network-Data Team.

- Port Reset configurations implemented and reviewed to ensure they match the requested requirements. (See below for port reset steps)

- Service request is updated with status and closed.

# Roles & Responsibility

Network Team – Implementing the changes in network

# Environment

Cisco equipment.

# References

NA

# Implementation Steps

- Find the cable id and data jack id details, switch and switch port details in the ticket.

- If the above information is not present in the ticket, assign the ticket to B&M to provide those details.

- Login to Wipro Jump box XXX.XXX.XXX.XXX

- Login to Network Jump box via mstsc to XXX.XXX.XXX.XXX

- Login to the given switch (mentioned in the ticket) for which the port needs to be reset.

- Execute command:

- Enable

- Enter password

- show interface (Number) status and check the status(connected/notconnect/err-disable)

- Execute following commands on Cisco switch/router:

config t

interface <Number>

shut

no shut

exit

exit

- Update the ticket with the updated interface status.

# Amendment History

| Version | Author(s) | Approved By | Nature of Change |
| --- | --- | --- | --- |
| 0.1 | Divya Kumari |  | Creation |
| 1.0 | Yogesh Rajguru |  | Reviewed & approved |

# Thank You